- PRINTING

- Technology:; Multi printhead thermal ink jet. - Printheads:; 10 semi-permanent printheads each with 208 nozzles (4 › black, 2 › cyan, 2 › magenta, 2 › yellow). - True resolution:; 600 › 600 dpi. - Minimum pen thickness:; .0032 inches. - Calibration:; Automatic printhead alignment Failing nozzle compensation Unattended printing in the overnight mode. - Size of ink tanks:; 400 ml. - Ink level monitoring:; Ink level indication Out of ink signaling. - Margins:; Leading and trailing strip: .2 inch Left and right sides: .12 inch. - Width detection:; Automatic. - Media types:; Uncoated (17–24#) Coated (24#) Polyester films (3–4 mil) Alternate photo glossy (50#) Vellum (16–20#). - Media Sources:; 1 or 2 front loadable rolls. - Media core size:; 2". - Maximum roll length:; 300 feet. - Output delivery:; Slide, front-accessible. - Noise level:; Less than 44 dB (A) standby Less than 65 dB (A) printing. - Dimensions:; 58" (h) › 77" (w) › 41" (d). - Electrical requirements:; 100/120/230 Volt 50/60 Hz. - Power consumption:; EPA-sleepmode (printer only)* 38 W Standby 108 W Run 159 W Note: Printer and Océ Power Logic® controller excluding monitor. - Weight:; 364–397 lbs.. - Safety approvals:; CE-compliance, TÜV-GS (IEC60950), UL, Cul (according to standard CAN/CSA C22.2 No 60950), CB Compliance with EN 60950. - Other:; Energy Star® compliant.

- SCANNING

| - Technology: |  | Fully integrated RGB color scanning |
| --- | --- | --- |
| - Cameras: |  | 3› Reduction type CCD Cameras |
| - Optical resolution: |  | 508 dpi |
| - Calibration: |  | Automatic color calibration based on IT8.7-card Automatic camera alignment Automatic stitching control |
| - Mechanical speed: |  | Max 8 ips |
| - Scanning width: |  | Max. 40" / 1016 mm |
| - Original thickness: |  | Max. 0.60 inch |
| - Original feed: |  | Face down |
| - Original out: |  | Rewind and hold Feed and release Feed and hold |
| - Original delivery: |  | Slide, front accessible Optional: original delivery rack at the back |
| - Dimensions: |  | 42" (h) › 53" (w) › 20" (d) |
| - Electrical requirements: |  | 100/120/230 Volt 50/60 Hz |
| - Power consumption: |  | Sleepmode: 45 W Stand-by: 135 W Run: 135W |
| - Warm up time: |  | From sleep mode: about 6 mi. From cold start (timer controllable) about 6 min. |
| - Weight: |  | 232 lbs. |
| - Safety approvals: |  | CE Compliance, TÜV-GS (IEC60950), UL, CB compliance with EN60950, EMC Compliance |

- COPYING

| - Copy processing technology: |  | Océ Image Logic® Color |
| --- | --- | --- |
| - Color management: |  | Based on OMP-profiles |
| - Number of copies: |  | 1–99 Set Memory: scan once, process once, print many |
| - Original handling: |  | Manual or automatic size detection Automatic Deskewing |
| - Job settings: |  | Programmable default setting Pre-programming of the next copy Single sheet or multiple sheet/set Priority setting on job level Start printing while scanning |
| - Copy color modes: |  | Grayscale, color, blueprint |
| - Copy quality modes: |  | Check, release, presentation Automatic resolution optimization |
| - Copy content modes: |  | Text & lines, Area fills |
| - Exposure control: |  | Manual fine adjustment Automatic background compensation (on/off ) |
| - Copy manipulation: |  | Leading strip adjustment (add or remove up to 16 inches) Invert Image mirroring Positioning Keep or delete region of interest |
| - Scale: |  | Manual: 10–1000 % Automatic: fit to media size fit to standard media format |
| - Roll/media selection: |  | Manual, Automatic |
| - Cut length: |  | Standard, Synchro or user defined |