Standard Accessories

Special Accessories a

t Extra Cost

Stepless table feeds & rapid system. Speed range covers both carbide & H.S.S. tools. Fixed or swivel head up to 45°. Suitable for Milling, Drilling, Facing, Boring & Grinding. Safety clutches for electrical & mechanical interlocking is provided for preventing trouble due to faulty operations. Automatic lifting & lowering of cross rail. Forced lubrication for guideways & gearboxes. Rapid operation to cross traverse of the milling head..

Extra Milling head on cross rail. Boring head on either of side columns. Grinding head on cross rail. Automatic locking of the cross rail. Extra milling head on either or both the columns. Bellow covers to the guideways. Optical read out. Right angle milling attachment.. Digital Read out...

| Plano Miller Special Type Milling Head | Plano Miller for machining of Lathe Beds |
| --- | --- |

Technical Specificaion of the Milling Head

| Model | 3 KW | 5 KW | 7.5 KW | 10 KW | 15 KW |
| --- | --- | --- | --- | --- | --- |
| Spindle Taper | ISO-40 | ISO-40 | ISO-50 | ISO-50 | ISO-50 |
| Spindle Speeds | 6 No. | 9 No. | 9 No. | 9 No. | 12 No. |
| Speed Range | 70 to 400rpm | 57 to 368rpm | 45 to 300rpm | 45 to 300rpm | 45 to 370 rpm |
| Quill Travel | 100 mm | 100 mm | 130 mm | 130 mm | 150 mm |
| Spindle Dia. | 80 mm | 85 mm | 100 mm | 120 mm | 150 mm |
| Head Swivel | + 45° | + 45° | + 45° | + 45° | + 45° |

1. TECHNICAL DESCRIPTION

1.1. BASIC MACHINE

1.1.1. INTRODUCTION

The  machine is intended for milling parts with large,  flat  surfaces  and  long slots as well as for simultaneous machining  of  more surfaces.  It  is fitted for machining workpieces of  steel  and  cast iron.

The  plano-miller consists of a cast iron bed. The  machine  table which  accommodates the workpiece to be machined slides along the  bed ways.  The  two  columns placed on each side of the  bed  are  rigidly secured  to the bed at their lower side and joined together through  a cross-beam,  at  their upper side. The bed, the two  columns  and  the cross-beam make-up a frame which offers to the machine high stiffness. The  right and left-hand horizontal milling heads and  the  cross-rail slide  along the columns guideways. The right and  left-hand  vertical milling heads move along the cross-rail longitudinal guideways.

The  machine controls are centralized on a pendant  control  panel located at the right-hand side of the machine. The machine can be also operated  from its left-hand side, since it is provided  with  another control panel with the respective controls.

The electrical equipment of the machine is placed inside an  electric  cabinet, nearby the machine, to the right-hand side of  the  machine.

1.1.2. MILLING HEADS

The milling heads carry out the main cutting movement. The gearbox is made up of a kinematic gear train consisting of heat treated  gears driven  by  an  electrical asynchronous motor of 30  kW.  The  working speeds  are  shifted with  manually driven  sliding   gears;  a  speed  range  from  12.5 to 630 rpm, in 18 steps, can be thus  obtained.  The main  spindle is carried in radial bearings with  cylindrical  rollers with a taper bore and precision axial ball bearings. The main  spindle is placed inside a quill having a travel of 300 mm. The quill feed can be  manually  achieved with a hand-wheel as well as  mechanically,  by means of an electrical motor; in the last case the quill feed rate  is 500 mm/min. The milling head has the possibility to rotate as  related to the normal (horizontal or vertical) position, to +_ 45 degrees. The milling head inclination is achieved with an electrical motor. A ruler and a vernier are provided for reading the inclination angle.

1.1.3. FEEDS OF THE MILLING HEADS

The feed movement of the horizontal milling heads along the guideways  of the columns is accomplished through a feedbox placed  at  the lower  side of the right-hand column, inside the column.  The  milling heads  can  be operated either simultaneously  or  independently.  The six-speed feedbox provided with sliding gears is driven by a two-speed asynchronous electrical motor. A feed range from 24 to 480 mm/min,  in 12  steps, can be obtained. The feed movement of the vertical  milling heads,  along  the  cross-rail guideways is performed by  means  of  a feedbox  placed at the right-hand end of the cross-rail. The  vertical milling heads can be also operated either simultaneously or  independently. The feedbox with sliding gears in 6 mechanical steps is  driven by a 2-speed asynchronous electrical motor and provides 12 feed steps, from 24 to 480 mm/min.

1.1.4. CROSS-RAIL

The cross-rail is a casting with a vertical rapid traverse of  600 mm/min.  The  rapid traverse is performed with  a  reducing  mechanism located inside the cross-beam.

The  movement  of the cross-rail as well as the  movement  of  the milling heads are performed by means of a trapezoidal thread screw and bronze nuts.

The  milling heads slides, the quills and the cross-rail are  provided with hydraulic interlocks. The hydraulic panel is placed on  the cross-rail, behind it. The working pressure is 100 daN/cm2.

1.1.5. BED AND TABLE

The bed and the table are heavily ribbed castings which offer high stiffness with time.

The  table feed is performed by means of a D.C.  electrical  motor and  an electronic servo-motor. The rotational movement is  translated into  translational movement by means of a worm and worm  rack  mechanism.  The  table working feed ranges is from 2 to 2000  mm/min,  with stepless adjustment during operation.

The  table feed mechanism is supplementarily provided with a  worm to take-up backlash at cut-down milling.

The table guideways are coated with a plastic material to diminish friction and increase the operating life of the guideways.

The lubrication is centralized and automatic. Graduated rules  and verniers of 0.1 mm accuracy are provided for measuring the strokes.

1.1.6. ELECTRICAL SYSTEM

The electrical equipment of the plano-miller consists of  driving, control  and protection conventional devices such as  driving  contactors, control contactors, time relay, thermal relays, fuses, automatic switches, etc.

The electrical equipment consists of :

- electrical cabinet;

- pendant control panels;

- electrical equipment placed on the machine.

The electrical cabinet has compartments which each accommodates  a counter panel with driving, control and protection equipment. The main switch  of the machine, which is operated with a handle from the  outside,  is  placed  on the front wall of the  electrical  cabinet.  The

cabinet doors are provided with microswitches which prevent the  cabinet from being energize                                                                                                                                                                  while one of the door is opened.

The  pendant  control panels contain the control,  signalling  and checking elements for the operator to command and surveil the  machine operating.  The machine controls can be initiated from either  of  the two control panels of the machine, that is either from the  right-hand side, or from the left-hand side.

The electrical equipment on the machine is made up of the sets  of connection  conductors  and  electromechanical  driving  and  checking elements,  such  as D.C. and A.C. motors, electromagnetic  brakes  and clutches,  electrovalves, pressure switches, level indicators,  microswitches. Connection couplings are provided for the electrical connections to the milling heads and the cross-rail.

The  whole electrical system has been designed and built so as  to provide the good operating of the machine in conditions of high safety and reliability.

1.1.7. COOLING SYSTEM

When  steel parts are machined, a cooling system is  required.  It consists of a tank located in the machine bed, a pump with a max. flow rate of 100 l/min and the related piping.

1.2. SPECIAL ACCESSORIES

1.2.1. Angular milling head

The angular milling head is mounted on one of the vertical milling heads  and  it  is driven by the last one. The  angular  milling  head facilitates the milling of vertical surfaces difficult to machine with the  4  standard heads of the machine as well as milling  of  surfaces impossible to be machined with these four standard heads, such as  the front faces of the long parts. The angular milling head has two ISO 60 tapers for tool clamping (at 180 degrees from each other) and has  the same speed range as the vertical milling head on which it is  mounted. The distance from the clamping flange to the main spindle of the angle milling head is 450 mm.

1.2.2. Indexing angular milling head

The  indexing  angular  milling head displays the  same  size  and features as the above mentioned one, with the only difference that  it can be indexed in 4 positions (4 x 90 degrees).

2.TECHNICAL DATA

| ###### Table |  |  |
| --- | --- | --- |
| - Width | mm | 1600 |
| - Useful length | mm | 4250 |
| - Total length | mm | 5930 |
| - Number of T-slots |  | 9(+2) |
| - Width of T-slots | mm(H7) | 28 |
| - Length between T-slots | mm | 145 |
| - Maximum load | kg | 15000 |
| - Maximum load | mm | 1045 |
| - Longitudinal travel | mm | 5185 |
| - Working feed rate, steples | s mm/min | 2-2200 |
| - Approaching feed rate | mm/min | 300 |
| - Positioning feed rate | mm/min | 2 |
| - Rapid traverse | mm/min | 4000 |
| - Driving motor torque | Nm | 90 |

| Milling heads |  |  |
| --- | --- | --- |
| - Number of vertical heads |  | 1 |
| - Number of horizontal heads |  | 2 |
| - Maximum vertical travel of the horizontal head | mm | 1250 |
| - Maximum horizontal travel of vertical head | mm | 2370 |
| Number of feed steps |  | 12 |
| - Inclination angle of heads | dgrs | +/-45 |
| - Working feed range | mm/min | 24-480 |
| - Rapid traverse of horizontal head | mm/min | 600 |
| - Rapid traverse of vertical head | mm/min | 860 |
| - Horizontal head feed motor power | kw | 10/13 |
| - Vertical head feed motor power | kw | 5,2/6,5 |
| - Main spindle diameter, in front bearing | mm | 160 |
| - Number of main spindle speed steps |  | 18 |
| Main spindle speed range | rpm | 12.5-630 |
| - Gearbox driving motor power (per each head) | kw | 30 |
| - Tool clamping taper according to DIN 2079 standard | ISO | 60 |
| - Quill diameter | mm | 270 |
| - Quill travel | mm | 300 |
| - Quill rapid traverse | mm/min | 500 |

| ###### Cross-rail |  |  |
| --- | --- | --- |
| - Vertical travel | - | 1300 |
| - Rapid traverse | mm/min | 600 |
| - Driving motor power | kw | 15 |

| ###### Functional distances |  |  |
| --- | --- | --- |
| - Length between columns | mm | 2050 |
| - Length from columns to table edge | mm | 225 |
| - Min/max length between the table surface and the horizon- tal head main spindle axis | mm | 0/1250 |
| - Min/max length from table surface to cross-rail | mm | 425/1600 |
| - Min/max length from the table surface to the frontal surface of the vertical head main spindles | mm | 0/1460 |

| ###### Overall dimensions |  |  |
| --- | --- | --- |
| - Length | mm | 12800 |
| - Width | mm | 5800 |
| - Height | mm | 5850 |
| - Approximate net weight | kg | 83000 |

| ###### Electric equipment |  |  |
| --- | --- | --- |
| Electric equipment | kw | 180 |
| - Operating voltage and frequency | 415 V/220 V +_ 10 % / 50 Hz |  |

3. DELIVERY SET

3.1.Basic machine, as described under item 1.1.

3.2.Standard accessories

The following standard accessories are included in the price  of the basic machine:

3.2.1. Spanners for servicing and maintenance of the machine

3.2.2. Hand pump for grease

3.2.3. Levelling blocks for machine setting on the foundation

3.3.Special accessories, as described under item 1.2.

The following accessories can be supplied against extra-price:

3.3.1. Angular milling head

3.3.2. Indexing angular milling head

3.3.3. Universal milling head

3.3.4. Chip conveyor

4.PAINTING

The  colour of the paint shall be decided upon according  to  the technology of Manufacturer and the request of the Buyer.