ED 1000PI; create printable version. Strung universal pt regimuri grele de lucru.

heavy industrial machine with cast iron machine bed. - wide (250mm) and inductively hardened as well as grinded machine bed - lead screw cover ensures a long durability - handy and easy operation via clearly designed control panel - removable bridge allows large turning diameters - all gliding machine elements can be fine adjusted via V-ledges - professional CAMLOCK D1-6 retainer - tail stock can be eccentrically adjusted for special works - oil bath gear lubrication with hardened, grinded gear wheels - huge spindle bore (DM-5mm) - food brake as well as an automatic brake system of the spindle - machine can handle a wide range of different threads without changing to different thread gear wheels - halogen lamp and cooling system standard inclus in dotare standard: universal cu 3 bacuri Ø200mm, universal cu 4 bacuri Ø250mm, platan frontal Ø350mm, reductie MT6/MT4, disp. automat cu arc pt filetare, suport scule pt. 4 cutite, dispozitiv de racire cu pompa, lampa halogen, set roti dintate de schimb, scule pt service, aparatoare mobile pt. protectie universal, aparatoare protectie stropire, disp. frinare la picior, 2 fixed centre points MT4, steady rest aperture 15-160mm, follow rest aperture 14-70mm, stand, revolving fence with fine adjustment function, accesorii: sistem de afisaj numeric 3-axis DRO, set scule pt strunjire 16mm, mandrina port burghiu MT4/B16, trusa de masurare si trasaj Diametrul maxim de strunjire peste ghidaje batiu; 410mm. Diametrul maxim de strunjire peste masa mobila; 250mm. Diametrul maxim de strunjire fara placutele de sub universal; 590mm. Diametrul alezajului din universal; 52mm. Distanta maxima la strunjire intre virfuri; 1000mm. Turatia arborelui principal; De la 45 la 1800 rot/min. Nr trepte de viteza; 16 trepte. Conul capului rotativ din universal; MT6. Sistemul de retinere si prindere in capul rotativ; Camlock ASA D1-6”. Gama de avansuri longitudinale; 0,05 – 1,7mm/rot.. Gama de avansuri transversale; 0,025 – 0,85mm/rot.. Cursa maxima de avans longitudinal; 120mm. Cursa maxima de avans transversal; 210mm. Filet normal– metric (39 pasi); 0,2 – 14mm/r. Filet englez - Inch (45 pasi); 72 – 2Gg/1. Filet trapezoidal (21 pasi); 8 – 44. Filet modular (18 pasi); 0,3 – 3,5. Conul papusii mobile; MT4. Cursa pinolei papusii mobile; 120mm. Dimensiune support port cutite ( 4 pozitii); 20mm. Putere motor principal; 5.5CP/4.5kW, 3faze 400V. Putere motor auxiliar. Dimensiuni de gabarit ( lungime x latime x inaltime); 2020x1590x1535mm. Greutate neta; 1800Kg..